|  | MPLS-based VPN | IPSec-Based VPN |
| --- | --- | --- |
| Service Model Applications | High-speed Internet services Business-quality IP VPN services E-commerce Application-hosting services Managed security Managed IP telephony Remote site backup |  |
| Scalability | Highly scalable—no site-to-site peering is required Capable of supporting tens of thousands VPNs over the same provider network | A very large IPSec-based VPN deployment requires supplemental planning and coordination to address key distribution, key management, and peering configuration Scalability becomes challenging for a very large, fully meshed IPSec VPN deployment |
| Network Service points | Service provider edge network | Customer premises equipment |
| Transparency | Resides at the network layer Transparent to applications |  |
| Provisioning | One-time provisioning of customer-edge and provider-edge devices to enable the site to become a member of an MPLS VPN group | In general, no network-level provisioning is required for CPE-based service offering Centralized provisioning for network-based service offering |
| Service deployment | Participating network elements at the core and edge must be MPLS-capable. Spread over different service providers if difficult. | Can be deployed across any existing IP network. Independent of service providers. |
| Session authentication | VPN membership established during provisioning, based on logical port and unique router descriptor Access to a VPN service group is defined during service configuration; | Via digital certificate or preshared key Packets that do not conform to the security policy are dropped |
| Confidentiality | Achieved via traffic separation, similar to technique used in trusted Frame Relay or ATM network environments. No encryption normally. | Via a flexible suite of encryption and tunneling mechanisms at the IP network layer. Normally traffic is encrypted. |
| QoS and SLAs | Achieved via scalable, robust QoS mechanism and traffic engineering capability in the service provider network. | Not addressed directly by IPSec. IPSec VPN deployments can preserve packet classification for QoS within an IPSec tunnel. |
| Client support | Not applicable; MPLS VPN is a network-based VPN service | IPSec VPN deployments. CPE has to support IPSec VPN. |
| User interaction | No user interaction required | For client-initiated IPSec VPN service offering, users need to interact with the IPSec client software. |
| Implementation | A VPN technology operating on a shared network. It is based on the logic of attaching tags to the IP packages coming from each site and creating isolated VPNs thanks to MPLS labels. | A VPN technology created as a result of transferring the data to be transmitted over shared networks (like the Internet) with the logic of end-to-end tunneling and encryption. |
| CPE VPN support | The customer premises equipment (CPE) do not need to have VPN support. | The customer premises equipment (CPE) need to have VPN support. |
| Additional Equipment Cost | There is no extra cost since there is no need to provide VPN support for CPEs VPN. | Usually there is no extra cost at the CPE. Some vendors, however, charge for additional IPSec license. |
| Service Cost | Higher service cost, due to the higher involvement of the service provider. | Normal cost for traditional Internet access. |
| Encryption | Natively there is no encryption. Customers “trust” the service provider. | Traffic is end-to-end encrypted and “ready” to pass through untrusted networks. |
| Tunneling | There is tunneling in transport layer, the branches can access each other directly or through the central site. Only “full-mesh” or “hub-n-spoke” topologies are possible and are subject to negotiate with the service provider. | In “hub-n-spoke” topology, the branches communicate with each other via the center, and by this customer has control of the traffic on the central site. In IPsec "fully meshed" topology, tunneling is made amongst all locations. It is possible to use “partial-meshed” topologies. All topologies are subject to customer’s choice, without any negotiations with the service provider. |
| Security of CPE | Usually the CPEs are not open to the Internet and security specific devices are not used. | CPEs are open to the Internet, they might also be exposed to attacks like the DoS (Denial of Service) attack. This requires a security specific appliance. |
| Data Confidentiality | BGP/MPLS VPNs seek to ensure data confidentiality by defining a Label Switched path between physical sites on a service provider network. | IPSec VPNs provide data confidentiality through robust encryption algorithms IPSec provides for better data confidentiality through encryption. |
| Data Integrity | There are no inherent methods to provide data integrity within BGP/MPLS VPNs. | IPSec uses hashing algorithms to ensure data integrity. |
| Data Availability | BGP/MPLS VPNs rely on LSPs for transport. A subject to SLA with the Service provider. | IPSec relies on the Internet for transport. General Internet access SLA. High availability can be achieved via redundant tunnels. |