# Tutorial: Serial Communication in Matlab

Objective:  This tutorial teaches you how to use Matlab serial objects to interact with external hardware.

Disclaimer:   The tutorial is not a replacement for reading the Matlab Documentation on Serial Interfacing; nor does it conver all the concepts and implementation details of serial communication and the RS-232 protocol.  The examples are soley geared toward the types of applications we see in our projects such as sending commands to control robots, motors, or reading sensors such as GPS, laser scanners, compasses, etc.   Other types of eqpitment may require different techniques not covered here.

Contact:

Associate Professor Joel M. Esposito

esposito@usna.edu

Systems Engineering Department

United States Naval Academy,

Annapolis, MD 21402

http://www.usna.edu/Users/weapsys/esposito/

# Hardware Devices

A lot of external hardware devices are designed to connect to a PC through a Serial Port.  Examples in our department include Koala Robots, iCreate Robots, Scorbot, Robix Kits; and lots of sensors such as the Sick or Hokuyo Laser Scanners, the Northstar kits, GPS, Compasses, etc

Traditionally all PC's had a serial port in the back.   However, now they are being replaced by USB ports.  "Serial ports" take many forms.  For example the Xbee modems create a wireless serial link.  Many laptops don't have a serial port – just USB ports.  USB to Serial Converters or Bluetooth Connections can function as "virtual serial ports", meaning that once set up correctly Matlab just veiws them as additional serrial ports.

## Basic Concepts

- Cabling:

- Serial Message:  You litterally send or recive data over this cable on a single pin as a series of bytes (1 byte = 8 bits or 0 – 255).

  - Example: [0]  [12] [27] [42] [112]

- Terminators:  Just as we use a period in English to dentote the end of a sentence, we use a "terminator" to indicate the end of a series of bytes that constitute a message.

  - The terminator can be anything the sender and receiver agree on but a "carrage return" (\r) is a common choice.

- Buffer:  If you don't understand how a buffer works, you will never understand serial communication.  Say a sensor is streaming back data to your program, more frequently than your program reads it.  On your computer this data gets stored in something called a buffer, until you decide to read it.  Think of a buffer as a list.

  - As new data values come in they get added to the bottom of the list (most recent data).

  - If your program reads a value from the buffer, it starts at the top of the list (oldest data).  Once you read a byte of data, it is no longer in the buffer; and the data in the second position on the list moves up to the top position, etc.

  - The buffer has a finite length (you set it).  This means there is a limit to how long the list can get.  Once the buffer is totally full, what happens when the sensor tries to send new data to the buffer?  The oldest data (top of the list) gets discarded forever, and all the entries move up, to make room on the bottom of the list for new data.

  - If you'rer smart about using the buffer, you can make sure you never miss any data.  If your not smart about it, it is easy to loose data or use old data.

Ex:   We create a buffer of length 5.   Initially it is empty

| 1 |  |
| --- | --- |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |

The sensor writes a value (10)  to it

| 1 | 10 |
| --- | --- |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |

The sensor writes another value (6) to it,  Note that the oldest data is in the first position and new data fills in the buffer from the bottom.

| 1 | 10 |
| --- | --- |
| 2 | 6 |
| 3 |  |
| 4 |  |
| 5 |  |

Now we read a value from it into the variable x.

| 1 | 6 |
| --- | --- |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |

x= 10.  Note that once we read the value it is no longer in the buffer!  Also note that we read the top element which was the oldest data.

Here is anothet scenario.   The buffer is full

| 1 | 10 |
| --- | --- |
| 2 | 6 |
| 3 | 12 |
| 4 | 3 |
| 5 | 1 |

Now the sensor writes a value to it (4)

| 1 | 6 |
| --- | --- |
| 2 | 12 |
| 3 | 3 |
| 4 | 1 |
| 5 | 4 |

The oldest data, in the top entry, (10) is discarded forever and all the entries shift up 1 spot, to make room for the new value (4).

- Message Length and Check Sum:  It's very possible, especially when you use wireless serial connections that individual bytes in the message can get lost or garbled.   There are a lot of complex schemes out there to check that this doesn't happen.  Here are two common ones that show up in some serial command interfaces.

  - One of the bytes in the message might indicate the length of the message (the total number of bytes the message should contain).

  - Another byte might be a checksum.   A checksum is a number computed using a simple arithmetic formula that can help you determine if the message has been garbled.  Here is an example:  CheckSum = 255 – Sum of all data bytes in message.

  - On the receiving end you can check that the length and checksum for the message you received match the actual message.  If they don't, you can decide to discard the message or request new data from the device.

  - Example:  [4]  [253] [1] [1] where 4 is the length (includes length and checksum bytes), [1] [1] are the data bytes (meaning depends on what the sensor does), and 253 = 255 – (1+1) is the checksum (does not include itself or length byte).

- Streaming vs. polling:

  - Polling a sensor is simple.   You send it a message requesting some data each time you want to take a measurement; then it returns a message containing that data.

  - Streaming means that you send the sensor a message to turn it on.   Then it begins sending back measurements as it gets them – usually at a regular interval

## What You Need to Know Before You Continue

Go the computer you will use, your device, and all the documentation that came with it.

- Find the Serial Port on the PC.   If there is none, use a USB-to-Serial Converter.   You may have to install drivers for it.  Connect the device to the PC.

- COM Port Number:   Each serial port on the PC is labeled COM1, COM2, etc.   You need to figure out which one you're attached to.

  - If there is a serial port at the back of the computer, chances are it's COM1.  However, even if there is only 1 serial port visible in the back, its possible its  COM2, or COM3 especially if there is a modem.

  - If you use a converter or blue tooth the port number gets assigned in software and may change each time you disconnect or reconnect the device.   On Windows XP (or vista classic view), go to Control Panel/System/Hardware/DeviceManager/Ports/Comm Ports and verify which port your device was assigned

- Device Settings:   Go to the documentation that came with your device and identify the Communication settings.  We'll need this for the next section.  Here is an example:

  - Baud Rate

- Serial Command Interface (SCI):   Go to the documentation that came with your device and find the serial command interface (may have a different name).   It explains how messages are formatted and how to interpret the results.  We'll need this later

# Setting up Serial port objects

## Basic Concepts

- Matlab uses a special variable type to keep track of serial connections – the Serial Object.

- Unlike nornal variables which have a single value, objects have many "attributes" or parameters that can be set. (ex. port number, baud rate, buffersize, etc)

- One of those attributes is the port number.  A label that cooresponds to which port your device is connected to.

- In order to actually send or recieve data through the serial port object it must be open.   When not in use it can be closed (not the same as deleting it)

- You can have many different serial objects in memory. They can all send and receive data at the same time as long as they are each on a different port.

- There can even be several objects associated with the same physical port.   However, only one of those objects associated with a given port can actually be open (sending or receiving data) at any time.

## Creating a Serial Port Object

Here is an example of how to do this.   the only piece of information you  must supply is which com port to use.   The rest of the attributes are set to some default values:

serialPort = serial('com1')

Serial Port Object : Serial-COM1

Communication Settings

Port:               COM1

BaudRate:           9600

Terminator:         'LF'

Communication State

Status:             closed

RecordStatus:       off

Read/Write State

TransferStatus:     idle

BytesAvailable:     0

ValuesReceived:     0

ValuesSent:         0

Note that this list of parameters and attributes it returns is not exhaustive.

## Setting the Parameters

Most of the time you don't want to use the default values.   You can view or change any attribute using the functions get and set.

get(serialPort, 'baudrate')

ans =

9600

set(serialPort, 'BaudRate', 19200)

get(serialPort, 'BaudRate')

ans =

19200

This method is cumbersome if you have a lot of things you want to change.   A better way to to set them when you create the Serial Object.

serialPort_new = serial('com1', 'baudrate', 19200, 'terminator', 'CR')

Serial Port Object : Serial-COM1

Communication Settings

Port:               COM1

BaudRate:           19200

Terminator:         'CR'

Communication State

Status:             closed

RecordStatus:       off

Read/Write State

TransferStatus:     idle

BytesAvailable:     0

ValuesReceived:     0

ValuesSent:         0

You can list as many properties as you want.   The name of the property goes in single quotes (check spelling) and the value follows (if the value is text then use single quotes)

## The Parameters

To see a list of parameters and their current values

get(serialPort)

ByteOrder = littleEndian

BytesAvailable = 0

BytesAvailableFcn =

BytesAvailableFcnCount = 48

BytesAvailableFcnMode = terminator

BytesToOutput = 0

ErrorFcn =

InputBufferSize = 512

Name = Serial-COM1

ObjectVisibility = on

OutputBufferSize = 512

OutputEmptyFcn =

RecordDetail = compact

RecordMode = overwrite

RecordName = record.txt

RecordStatus = off

Status = closed

Tag = GarminGPS

Timeout = 0

TimerFcn =

TimerPeriod = 1

TransferStatus = idle

Type = serial

UserData = []

ValuesReceived = 0

ValuesSent = 0

SERIAL specific properties:

BaudRate = 19200

BreakInterruptFcn =

DataBits = 8

DataTerminalReady = on

FlowControl = none

Parity = none

PinStatus = [1x1 struct]

PinStatusFcn =

Port = COM1

ReadAsyncMode = continuous

RequestToSend = on

StopBits = 1

Terminator = LF

Note that some values are just numbers, while others can only take on certain values in a list ( ex. 'on' or 'off ').  To see a list of all paremeters with valid choices type (note that the curly brace denotes the default value)

set(serialPort)

ByteOrder: [ {littleEndian} | bigEndian ]

BytesAvailableFcn: string -or- function handle -or- cell array

BytesAvailableFcnCount

BytesAvailableFcnMode: [ {terminator} | byte ]

ErrorFcn: string -or- function handle -or- cell array

InputBufferSize

Name

ObjectVisibility: [ {on} | off ]

OutputBufferSize

OutputEmptyFcn: string -or- function handle -or- cell array

RecordDetail: [ {compact} | verbose ]

RecordMode: [ {overwrite} | append | index ]

RecordName

Tag

Timeout

TimerFcn: string -or- function handle -or- cell array

TimerPeriod

UserData

SERIAL specific properties:

BaudRate

BreakInterruptFcn: string -or- function handle -or- cell array

DataBits

DataTerminalReady: [ {on} | off ]

FlowControl: [ {none} | hardware | software ]

Parity: [ {none} | odd | even | mark | space ]

PinStatusFcn: string -or- function handle -or- cell array

Port

ReadAsyncMode: [ {continuous} | manual ]

RequestToSend: [ {on} | off ]

StopBits

Terminator

## Suggestions on Parameters

Some of these you don't really need to change.  Others you will want to change.

### Always Set

You always have to set this to match what is specified in the documentation that came with your device.

- BaudRate

### Always Check

The defaults here are usually OK, but you should check that they match whatever is specified in the device documentation.

- Terminator (sometimes have to change)  'LF' is linefeed, 'CR' is carrage return, etc

- FlowControl (defaults usually OK)

- Parity (defaults usually OK)

- DataBits (defaults usually OK)

- ByteOrder (more on this later)

### Good Idea To Set

Your device will work without setting these but you can set these to make your life easy later.

- Tag:   The tag is like giving the serial port object a nickname.   If have a few different serial ports open this a good way to keep track of them.   Example, serialPort is configured to talk with a garmin GPS.

set(serialPort, 'tag', 'GarminGPS')

- TimeOut:   If you try to read data from the serial port and there is no data in the buffer  matlab will keep trying to read for "Timeout" seconds (default 10 sec):

get(serialPort, 'Timeout')

ans =

10

This might really slow down your code.    There are ways around this, but if there is no data there you probably don't want to sit there for 10 seconds, so consider making it smaller.  On the other hand, it does take some time for messages to pass over the wire, so setting it to zero means you will probably miss a lot of messages.

- InputBufferSize:   This specifies how long the buffer is.   The default is 512 bytes.   That might not be long enough for your messages.   Especially if you think the sensor will be streaming data back more frequently than you plan on reading the buffer.    Remember if the sensor tries to send data and the buffer is full it will discard some old data and it will be gone forever.   On the otherhand, having an unessecarliy large buffer can be cumbersome.

# Closing Serial Port Objects

## Concepts

When your done with a serial port object it doesn't go away.   Also, closing it, deleting it from memory and clearing it from the workspace are three separate actions.

## Example Code

For technical reasons you have to use this systax to properly get rid of it:

delete(serialPort_new)

clear serialPort_new

# Comprehensive Example On Creating a New Port

Sometimes, if your program does not terminate correctly you have to abort  (CTRL-C) before you can properly delete or close a port.   So, it is good practice to check for old serial objects before creating new ones:

oldSerial = instrfind('Port', 'COM1');  % Check to see if there are existing serial objects (instrfind) whos 'Port' property is set to 'COM1'

% can also use instrfind() with no input arguments to find ALL existing serial objects

if (~isempty(oldSerial))  % if the set of such objects is not(~) empty

disp('WARNING:  COM1 in use.  Closing.')

delete(oldSerial)

end

% creating a new serial object for my GPS (note I can do all this in one line if I wanted to

serGPS = serial('COM1');  % Define serial port

set(serGPS, 'BaudRate', 4800);  % instructions for GPS gave me this

set(serGPS, 'Tag', 'GPS');   % give it a name for my own reference

set(serGPS, 'TimeOut', .1);  %I am willing to wait 0.1 secs for data to arive

% I wanted to make my buffer only big enough to store one message

set(serGPS, 'InputBufferSize',  390 )

get(serGPS) %so you can see my result

WARNING:  COM1 in use.  Closing.

ByteOrder = littleEndian

BytesAvailable = 0

BytesAvailableFcn =

BytesAvailableFcnCount = 48

BytesAvailableFcnMode = terminator

BytesToOutput = 0

ErrorFcn =

InputBufferSize = 390

Name = Serial-COM1

ObjectVisibility = on

OutputBufferSize = 512

OutputEmptyFcn =

RecordDetail = compact

RecordMode = overwrite

RecordName = record.txt

RecordStatus = off

Status = closed

Tag = GPS

Timeout = 0.1

TimerFcn =

TimerPeriod = 1

TransferStatus = idle

Type = serial

UserData = []

ValuesReceived = 0

ValuesSent = 0

SERIAL specific properties:

BaudRate = 4800

BreakInterruptFcn =

DataBits = 8

DataTerminalReady = on

FlowControl = none

Parity = none

PinStatus = [1x1 struct]

PinStatusFcn =

Port = COM1

ReadAsyncMode = continuous

RequestToSend = on

StopBits = 1

Terminator = LF

# Writing To The Serial Port

Before you can write to your serial port, you need to open it:

fopen(serGPS)

Now you need to figure out two things from the Serial Command Interface (SCI)  that came with your device:

- Will you send binary data (bytes) or text (ascii)?

- What will you send to it?

If your SCIs messages look like a list of numbers (ex: [4][253][1][1])), its probably the first choice.   Note that even though what you send is actually binary, the documentation might list it as numbers between 0 and 255, or hexidecimal numbers.

If your SCIs messages look like a mix of text and numbers (ex:  'MOVE 31'), its probably the second choice.

## Writing Binary Data

Use the command fwrite to send four bytes of binary data

fwrite(serGPS, [0, 12, 117, 251]);

## Writing ASCI Commands

Use the command fprintf to send asci data.   You can use a mix of text in single quotes and variables values.

moveNum = 98;  pauseTime = 2; % just some example data

fprintf(serGPS, 'MOVE %d, PAUSE %d', [moveNum, pauseTime] ) ;  % note

Its important to understand that a number, (ex. 98) is not sent as a number.   Its actually the asci code for the characters '9' and '8'.

## Example Code

# Reading From The Serial Port

## Streaming vs Polling:   Flushing the Buffer

If you are going to poll the device (send it a request each time you want to get data) you don't want to read any old data that might be left over in the buffer.   This is a useful and quick way to clean it out

N = serRoomba.BytesAvailable();

while(N~=0)

fread(serRoomba,N);

N = serRoomba.BytesAvailable();

end

## Reading Formatted ASCI

Say my device returns a text sentence like this

X, 2.1, Y, 3.2, T, -0.5

Where X is the x position, Y the y position and T is the heading.

sentence = fscanf(serialObj, '%s');  % this reads in as a string (until a terminater is reached)

[x, xPosition, y, yPosition, t, Heading] = strread(sentence, '%s%f%s%f%s%f', 1, 'delimiter', ',')

% decodes "sentence" as string, float, string, float, string, float"  seperated (delimted) by commas

ans =

x  = 'X'

xPosition = 2.1

y = 'Y'

yPosition  = 3.2

t = 'T'

Heading = -0.5

## Reading Data

You can use fread to read in data (not text).   It can automatically format the data for you.   Here is an example.  Say the buffer currently has 2 bytes of data in it

[1], [8]

a = fread(serialObj, 2);

%   Will read two bytes  and create a vector

a = [1; 8]

If you omit the ,2 the result would be the same -- it will just read until either a terminater is reached or there is no more data in the buffer

Alternatively, suppose you know those two bytes are used to express a singel 16-bit integer, you can use

a = fread(serialObj, 1, 'int16')

ans =

264

Which is equivilent to 1*(256^1) + 8*(256^0)

Note that even though we only "read" 1 value, two elements were take out of the buffer since a 16 bit interger is actually composed of two bytes.   To see a list of all format types, type

>>  help serial/fread

# Putting It All Together

## Good Programming Advice

- to make your code crash resistance try: preinitializating any return variables, then  put the reading and writiing  to the serial port inside a  T RY-CATCH statement

- Debugging??

## Example 1:  GPS Initialization

function [serGPS] = initializeGarmin(ComPortNumber)

%This function initializes the serial port properly

% for use with the Garmin GPS

%COMPORT is the number of the serial port:   ex use 3 for 'COM3'

% port number can be checked in

% Control Panel/System/Hardware/DeviceManager/Ports

% serGPS output is a matlab serial port object

% Esposito 6/25/2008 with code from regneier, bishop, et al; modified by

% MIDN 1/C Li

port = strcat('COM',num2str(ComPortNumber) );

out = instrfind('Port', port);  % Check to see if THAT serial port is already defined in MATLAB

if (~isempty(out))  % It is

disp('WARNING:  port in use.  Closing.')

if (~strcmp(get(out(1), 'Status'),'open'))  % Is it open?

delete(out(1)); % If not, delete

else  % is open

fclose(out(1));

delete(out(1));

end

end

serGPS = serial(port);  % Define serial port

set(serGPS, 'BaudRate', 19200);  % Default Baud rate of 19200

set(serGPS, 'Tag', 'GPS');   % give it a name

set(serGPS, 'TimeOut', .1);

% want to make the buffer big enough that new message can always fit

%(example is 389 characters long but messsage length is variable)

% but not so big as to hold 2 messages

set(serGPS, 'OutputBufferSize', floor(389*1.5) );

set(serGPS, 'InputBufferSize',  floor(389*1.5) );

fopen(serGPS);  % Open it;

pause(1)  % give it a second to start getting data

disp('Garmin Initialized')

## Example 1:   Read  GPS

function [lat, lon] = ReadGarmin(serGPS)

% Reads a streaming GPS.   If no data, does not wait, returns nans.

%  Note that this code never crashes.   Even if the GPS unit dies or gets unplugged, or cant find satelites.

% serial port must first be initialized using initializeGarmin

% inputs: serGPS (from initializeGarmin)

% Outputs:  x (lon) and y(lat)

%%  initialize to nan, will have something to return even if serial comms fail

lat =nan; lon = nan;

%% IF THERE IS NO DATA?

if (get(serGPS, 'BytesAvailable')==0)

disp('Data not avail yet.   Try again or check transmitter.')

return

end

%% IF THERE IS DATA

while (get(serGPS, 'BytesAvailable')~=0)

try

% read until terminator

sentence = fscanf(serGPS, '%s');

Ns = length(sentence);

% Make sure header is there

if strcmp(sentence(1:6),'$GPRMC')

[prefixRMC,timeRMC,ActiveRMC,lat,latdirRMC,lon,londirRMC,spdKnots,AngleDeg] = strread(sentence, '%s%f%s%f%s%f%s%f%f', 1, 'delimiter', ',');

%  these cases mean that the GPS can't find satelities

%  (status not equal to A (active)

% or the sentence wasn't long enough to fill in lat and lon

if isempty(lat)||(ActiveRMC{1} ~='A')

lat = nan;

end

if isempty(lon)||(ActiveRMC{1} ~='A')

lon = nan;

end

end

catch ERR_MSG

% if something didn't work correctly the error message displays

disp('Error Reading Data! Check Unit')

end

end

## Example 2:  Initialize the iRobot Create

function [serPort] = RoombaInit(my_COM);

% initializes serial port for use with Roomba

% COMMport is the number of the comm port

% ex. RoombaInit(1) sets port = 'COM1'

% note that it sets baudrate to a default of 57600

% can be changed (see SCI).  Will NOT work if robot is plugged into

% charger.

% An optional time delay can be added after all commands

% if your code crashes frequently

global td

td = 0.01;

% This code puts the robot in SAFE(130) mode, which means robot stops

% when cliff sensors or wheel drops are true; or when plugged into charger

Contrl = 132;

% Esposito 9/2008

warning off

%% set up serial comms,

% output buffer must be big enough to take largest message size

comm = strcat('COM', num2str(my_COM));

a = instrfind('port',comm);

if ~isempty(a)

disp('That com port is in use.   Closing it.')

fclose(a)

delete(a)

end

disp('Establishing connection to Roomba...');

% defaults at 57600, can change

serPort = serial(comm,'BaudRate', 57600);

set(serPort,'Terminator','LF')

set(serPort,'InputBufferSize',100)

set(serPort, 'Timeout', 1)

set(serPort, 'ByteOrder','bigEndian');

set(serPort, 'Tag', 'Roomba')

disp('Opening connection to Roomba...');

fopen(serPort);

%% Confirm two way connumication

disp('Setting Roomba to Control Mode...');

% Start! and see if its alive

Start=[128];

fwrite(serPort,Start);

pause(.1)

fwrite(serPort,Contrl);

pause(.1)

% light LEDS

fwrite(serPort,[139 25 0 128]);

% set song

fwrite(serPort, [140 1 1 48 20]);

pause(0.05)

% sing it

fwrite(serPort, [141 1])

disp('I am alive if my two outboard lights came on')

confirmation = (fread(serPort,4))

pause(.1)

## Example 2:  Read all sensors from the Create.

function [BumpRight, BumpLeft, BumpFront, Wall, virtWall, CliffLft, ...

CliffRgt, CliffFrntLft, CliffFrntRgt, LeftCurrOver, RightCurrOver, ...

DirtL, DirtR, ButtonPlay, ButtonAdv, Dist, Angle, ...

Volts, Current, Temp, Charge, Capacity, pCharge]   = AllSensorsReadRoomba(serPort);

% Reads all 23 Roomba Sensors from a single data packet.   Values are

% [BumpRight (0/1), BumpLeft(0/1), BumpFront(0/1), Wall(0/1), virtWall(0/1), CliffLft(0/1), ...

%    CliffRgt(0/1), CliffFrntLft(0/1), CliffFrntRgt(0/1), LeftCurrOver (0/1), RightCurrOver(0/1), ...

%    DirtL(0/1), DirtR(0/1), ButtonPlay(0/1), ButtonAdv(0/1), Dist (meters since last call), Angle (rad since last call), ...

%    Volts (V), Current (Amps), Temp (celcius), Charge (milliamphours), Capacity (milliamphours), pCharge (percent)]

% Can add others if you like, see code

% Esposito 3/2008

warning off

global td

sensorPacket = [];

% flushing buffer

confirmation = (fread(serPort,1));

while ~isempty(confirmation)

confirmation = (fread(serPort,26));

end

%% Get (142) ALL(0) data fields

fwrite(serPort, [142 0]);

%% Read data fields

BmpWheDrps = dec2bin(fread(serPort, 1),8);  %

BumpRight = bin2dec(BmpWheDrps(end))  % 0 no bump, 1 bump

BumpLeft = bin2dec(BmpWheDrps(end-1))

if BumpRight*BumpLeft==1  % center bump sensor is really just

% left AND right at same time

BumpRight =0;

BumpLeft = 0;

BumpFront =1;

else

BumpFront = 0;

end

Wall = fread(serPort, 1)  %0 no wall, 1 wall

CliffLft = fread(serPort, 1) % no cliff, 1 cliff

CliffFrntLft = fread(serPort, 1)

CliffFrntRgt = fread(serPort, 1)

CliffRgt = fread(serPort, 1)

virtWall = fread(serPort, 1)%0 no wall, 1 wall

motorCurr = dec2bin( fread(serPort, 1),8 );

Low1 = motorCurr(end);  % 0 no over current, 1 over Current

Low0 = motorCurr(end-1);  % 0 no over curr, 1 over Curr

Low2 = motorCurr(end-2);  % 0 no over curr, 1 over Curr

LeftCurrOver = motorCurr(end-3)  % 0 no over curr, 1 over Curr

RightCurrOver = motorCurr(end-4)  % 0 no over curr, 1 over Curr

DirtL = fread(serPort, 1)

DirtR = fread(serPort, 1)

RemoteCode =  fread(serPort, 1); % coudl be used by remote or to communicate with sendIR command

Buttons = dec2bin(fread(serPort, 1),8);

ButtonPlay = Buttons(end)

ButtonAdv = Buttons(end-2)

Dist = fread(serPort, 1, 'int16')/1000 % convert to Meters, signed, average dist wheels traveled since last time called...caps at +/-32

Angle = fread(serPort, 1, 'int16')*pi/180 % convert to radians, signed,  since last time called, CCW positive

ChargeState = fread(serPort, 1);

Volts = fread(serPort, 1, 'uint16')/1000

Current = fread(serPort, 1, 'int16')/1000 % neg sourcing, pos charging

Temp  =  fread(serPort, 1, 'int8')

Charge =  fread(serPort, 1, 'uint16') % in mAhours

Capacity =  fread(serPort, 1, 'uint16')

pCharge = Charge/Capacity *100  % May be inaccurate

%checksum =  fread(serPort, 1)

pause(td)